Measure Wind Speed

Build an Anemometer

Materials:

A ping pong ball		A small screw		A piece of string

A plastic protractor		A ruler				Strong tape

Make the anemometer:

- Measure out a 30 cm piece of string.

- Twist the screw eye into the Ping-Pong ball and tie one end of the string to it.

- Tape the other end of the string to the center of the straight edge of the protractor.

- Place the protractor so that it rests against the edge of the ruler with the curved side pointing downwards and tape them together.

Use the anemometer and a wind speed conversion chart:

- Take the anemometer outdoors and tape the ruler to a post or a flat surface so the protractor is extended beyond the post or surface.

- Check to see that the string with the Ping-Pong ball is still hanging at the center mark.

- To measure the wind speed, first shield the anemometer from the wind with a person’s body.  If the string doesn’t hang directly at 90degree mark, straighten the anemometer.  Now step away enough to let the wind hit the anemometer.

- As the wind blows the Ping-Pong ball, look at the number on the protractor that the string stretches across.  Use the closest whole number and record it.

- Find this number on the Wind Speed Conversion chart under the column called Angle Degrees. Then read the number directly across from it under the column Wind Speed in Miles per Hour and record.  This tells how fast the wind is blowing.

- Repeat steps 2 – 5 two more time to find the average wind speed.

| Wind Speed Conversion Chart |  |
| --- | --- |
| Angle Degrees | Wind Speed in MPH |
| 90 | 0 |
| 85 | 5 |
| 80 | 8 |
| 75 | 10 |
| 70 | 11 |
| 65 | 13 |
| 60 | 15 |
| 55 | 16 |
| 50 | 18 |
| 45 | 20 |
| 40 | 21 |
| 35 | 23 |
| 30 | 25 |
| 25 | 28 |
| 20 | 32 |
| *To find the wind speed in kilometer per hour, multiply the wind speed by 1.609. |  |